ADAPTIVE MULTI-JUNCTION TRAFFIC CONTROLLER (AMTGC)

TASK 6 — FINAL TECHNICAL REPORT

ElevanceSkills Internship Project | Verilog HDL | AMD Xilinx Vivado 2022.2

# 1. Abstract

The Adaptive Multi-Junction Traffic Controller (AMTGC) is a Verilog HDL digital design intended to coordinate traffic signals at two four-way junctions. The architecture uses reusable FSM-based junction controllers and a generic timer. Its principal features are traffic-density-based adaptive green timing, pedestrian request handling through a shared arbiter, green-wave coordination between Junction A and Junction B, and an emergency override that forces the vehicle signals to an all-red condition. The design was developed and verified through behavioral simulation in AMD Xilinx Vivado 2022.2.

# 2. Objectives

Develop a modular and parameterized RTL design for two traffic junctions.

Control North–South and East–West signal phases using finite-state machines.

Adjust green duration according to traffic-density input while enforcing configured minimum and maximum limits.

Coordinate pedestrian requests through a shared arbitration module.

Provide green-wave timing coordination between Junction A and Junction B.

Force a safe all-red vehicle-signal condition during emergency override.

Verify the design with testbenches and preserve simulation evidence for review.

# 3. System Architecture

The top-level module amtgc_top integrates two instances of junction_controller, one for each junction. Each controller uses generic_timer for phase timing. The shared ped_arbiter coordinates pedestrian grants. Top-level logic distributes traffic-density inputs, start/coordination signals, and the emergency override, and exposes the signal outputs for both junctions.

Architecture diagram: place the final architecture/block diagram from the docs folder here if a combined report copy is needed. Ensure that the diagram reflects the final RTL hierarchy.

# 4. RTL Modules

| Module / file | Responsibility | Engineering notes |
| --- | --- | --- |
| amtgc_top.v | Top-level integration of both junctions | Connects junction controllers, pedestrian arbitration, coordination, and emergency control. |
| junction_controller.v | Signal sequencing FSM | Controls green, yellow, red-clearance, wait, and pedestrian states. |
| generic_timer.v | Reusable phase timer | Parameterized counter-based timer; retain the version used by the passing verification run. |
| ped_arbiter.v | Shared pedestrian request arbitration | Selects/grants pedestrian requests; fairness should be demonstrated by testbench results. |
| amtgc_top_tb.v / task testbenches | Simulation verification | Applies test scenarios and checks expected controller behavior. |

# 5. Junction Controller FSM

The supplied junction_controller.v contains eight states: S_WAIT, S_NS_GREEN, S_NS_YELLOW, S_RED_NS_EW, S_EW_GREEN, S_EW_YELLOW, S_RED_EW_NS, and S_PED. The normal sequence serves North–South traffic, transitions through yellow and red clearance, serves East–West traffic, and then returns through red clearance. At a red-clearance decision, a pending pedestrian request can route the FSM to S_PED. After the pedestrian timer completes, the controller returns to the direction selected by ped_return_to_ns.

Emergency behavior: when emergency_override is asserted, the next-state logic selects S_WAIT and output decoding forces ns_green, ns_yellow, ew_green, and ew_yellow low; all_red is asserted and ped_walk is deasserted.

FSM diagram: include the final junction_controller_fsm.png from docs. Review every transition label against the submitted Verilog before final delivery.

# 6. Adaptive Green Timing

The controller calculates the green target using MIN_GREEN_TIME + traffic_density × GREEN_EXTENSION. The result is clamped between MIN_GREEN_TIME and MAX_GREEN_TIME. The target is sampled as the controller enters a green phase and is used for either NS or EW green timing. Yellow, red-clearance, and pedestrian phases use their dedicated parameters.

Separating the timer into generic_timer makes the phase timing reusable and keeps timing logic distinct from the FSM. The verified generic_timer source should not be changed without rerunning the full verification suite.

# 7. Pedestrian Handling

The junction controller records a granted pedestrian request in ped_pending. At a red-clearance state, the FSM checks whether a request is pending and enters S_PED when required. Once the pedestrian phase timer completes, the pending request is cleared and the controller resumes the appropriate traffic direction. The system-level ped_arbiter coordinates requests from the two junctions. Claims about arbitration fairness should be supported by the actual testbench checks and simulation log.

# 8. Green-Wave Coordination

Green-wave coordination aims to establish a configured timing relationship between the start of a compatible green phase at Junction A and Junction B. The project testbench previously reported a measured delay of 50,000 ns and a PASS result against its stated minimum-delay condition. Before publishing this numeric result as a final engineering claim, verify the time units, clock period, and threshold in the final testbench and preserve the matching simulation log.

# 9. Emergency Override and Safety

The emergency override has priority over normal output decoding. When active, all vehicle green and yellow outputs are forced low, all_red is asserted, and ped_walk is disabled. The timer reset is also combined with emergency_override in the supplied junction controller. The final verification evidence should show that both junctions enter the all-red condition and remain there for the duration of the override.

# 10. Verification Strategy and Results

The reported Task 5 testbench result was 361 checks performed, zero detected errors, and PASS for all enabled checks. The recorded simulation output also indicated pedestrian grants of 100 for Junction A and 100 for Junction B, emergency phase coverage of 8/8, and simulation completion at 6520 ns. These figures are included from the project results shared during development; confirm that the archived log and screenshots correspond to the exact RTL files being delivered.

Inspect compile and simulation messages for errors or warnings that affect functionality.

Review waveforms for normal NS/EW sequencing and yellow/red clearance phases.

Check adaptive green duration at representative density values and at the minimum/maximum limits.

Check that pedestrian requests are granted and serviced at both junctions.

Check the green-wave timing condition using the final testbench's exact units and threshold.

Assert emergency_override and confirm all-red outputs at both junctions.

Re-run verification after any RTL or testbench change and update the archived evidence.

# 11. Debugging Notes and Design Trade-offs

During testbench development, a recurring issue was simulation ending before all planned density cases had completed. This indicates that stimulus duration, phase progression, and timeout settings must be considered together; a timeout should be long enough for the full sequence but finite enough to detect a stalled design. The measurement task should wait for a fresh NS-green rising edge and the corresponding falling edge, and density should be changed at a point that allows the controller to sample it for the intended next phase.

The green-wave test also requires careful interpretation of time units. A log value in nanoseconds must be compared with a threshold expressed in the same units; the clock period and configured cycle count should be stated explicitly. Avoid treating a PASS printout alone as proof if the measured value and threshold appear to use inconsistent scales.

The principal trade-offs are modularity versus integration complexity, adaptive responsiveness versus predictable phase duration, and pedestrian responsiveness versus traffic-flow continuity. Reusing a generic timer and controller module reduces duplication, while centralized pedestrian arbitration makes shared-request handling explicit.

# 12. Limitations and Future Improvements

Validate the design on FPGA hardware before considering real-world deployment.

Connect calibrated traffic sensors rather than relying only on externally supplied density values.

Document the green-wave delay parameter and units explicitly at the top-level interface.

Add assertions for mutually exclusive vehicle green signals and bounded emergency response.

Expand boundary-value and randomized testing for density, reset, pedestrian requests, and emergency overlap.

Add automated regression scripts and a single summary of pass/fail results.

Record a demonstration video showing normal traffic, adaptive timing, pedestrian handling, green-wave coordination, and emergency override.

# 13. Conclusion

AMTGC demonstrates a modular Verilog RTL approach to coordinating two traffic junctions using reusable FSM controllers and a generic timer. The main capabilities are adaptive green timing, pedestrian request handling, green-wave coordination, and emergency all-red behavior. The reported Task 5 verification completed 361 checks with zero detected errors. A professional handoff requires preserving the exact verified RTL, keeping diagrams aligned with the implemented transitions, and archiving the testbench, logs, waveforms, and documentation in a clear repository structure.

# Appendix A — Final Delivery Checklist

rtl/ contains the final verified RTL source files only.

tb/ contains the final testbench files.

docs/ contains Task 1–5 reports, architecture/block diagram, FSM diagram, and this Task 6 report.

sim/ contains final waveforms, PASS screenshots, and relevant simulation logs.

README.md describes the real folder structure and provides reproducible Vivado steps.

The final RTL is identical to the version used for the passing Task 5 verification.

The demonstration video is recorded and linked or stored as required by the internship.