ELEVANCESKILLS INTERNSHIP
TASK 2 – PARAMETERIZED RTL MODULE DESIGN
Adaptive Traffic Controller

1. Objective:

The objective of this task is to design reusable and parameterized RTL modules for an adaptive traffic controller using Verilog HDL. The design includes a configurable generic timer and a parameterized junction controller implemented using a Moore finite state machine (FSM). The same junction controller is reused for two traffic junctions with different timing parameters. The complete design is verified through simulation using Xilinx Vivado 2022.2.

2. Task Requirements:

Design a reusable parameterized generic timer module.

Design a parameterized junction controller using a Moore FSM.

Support North-South Green, North-South Yellow, All-Red, East-West Green, East-West Yellow and Pedestrian phases.

Use timing parameters instead of hardcoded timing values.

Instantiate the same junction controller for Junction A and Junction B with different parameter values.

Verify the modules using independent testbenches and Vivado simulation.

Verify parameterization using multiple timer target values and different junction timing parameters.

Maintain safe all-red gaps between traffic directions.

3. Design Architecture:

The design is divided into reusable RTL blocks. The generic_timer provides configurable timing and a completion pulse. The junction_controller uses the timer and a Moore FSM to generate traffic-light outputs. The traffic_two_junction top-level module instantiates two copies of the same junction_controller with different parameters.

Clock and reset are supplied to the controllers.

Each junction controller receives a pedestrian request.

The junction controller selects the active traffic phase using its FSM.

The generic timer determines when the current phase is complete.

Junction A and Junction B use the same RTL structure but different parameter values.

4. Module 1 – Generic Timer:

The generic_timer is an independent, reusable timing module. Its COUNTER_WIDTH parameter controls the counter size. The count_target input determines the target count. When the target is reached, done produces a completion pulse. The timer also supports reset during counting and a zero-target corner case.

4.1 Interface:

| Signal | Direction | Width | Purpose |
| --- | --- | --- | --- |
| clk | Input | 1 | System clock |
| reset | Input | 1 | Synchronous reset |
| start | Input | 1 | Enables timer |
| count_target | Input | COUNTER_WIDTH | Target count |
| done | Output | 1 | Completion pulse |
| count | Output | COUNTER_WIDTH | Current count |

4.2 Parameterization:

The timer uses the COUNTER_WIDTH parameter. The testbench verifies target values 3, 5 and 0 without modifying  the timer RTL.

5. Module 2 – Junction Controller:

The junction_controller is a parameterized Moore FSM. Traffic-light outputs depend only on the current FSM state. The controller contains separate states for traffic phases and safe all-red intervals, together with an optional pedestrian phase.

5.1 FSM States:

| State | Description | Output |
| --- | --- | --- |
| S_NS_GREEN | North-South traffic | ns_green |
| S_NS_YELLOW | North-South warning | ns_yellow |
| S_ALL_RED_BEFORE_EW | Safety interval | all_red |
| S_EW_GREEN | East-West traffic | ew_green |
| S_EW_YELLOW | East-West warning | ew_yellow |
| S_ALL_RED_BEFORE_NS | Safety interval | all_red |
| S_PED | Pedestrian crossing | ped_walk |
| S_ALL_RED_AFTER_PED | Safety interval | all_red |

5.2 Parameters:

GREEN_TIME – green duration.

YELLOW_TIME – yellow duration.

RED_TIME – all-red safety duration.

PED_TIME – pedestrian duration.

COUNTER_WIDTH – internal timer width.

6. Two-Junction Integration:

The traffic_two_junction module instantiates the same junction_controller twice. Junction A and Junction B use different timing parameters.

| Parameter | Junction A | Junction B |
| --- | --- | --- |
| GREEN_TIME | 3 | 4 |
| YELLOW_TIME | 2 | 2 |
| RED_TIME | 2 | 3 |
| PED_TIME | 2 | 3 |
| COUNTER_WIDTH | 8 | 8 |

This demonstrates that the same reusable RTL module can support different junction requirements without duplicating the controller design.

7. Verification and Testbenches:

generic_timer_tb.v – verifies multiple target values, reset behavior and the zero-target case.

junction_controller_tb.v – verifies the traffic phase sequence and pedestrian request handling.

traffic_two_junction_tb.v – verifies both junction instances and their different timing behavior.

8. Simulation Results:

Simulation was performed in Xilinx Vivado 2022.2. The waveforms confirm operation of the timer, junction FSM, pedestrian phase and two-junction parameterization.

8.1 Generic Timer Waveform:

Targets 3, 5 and 0, reset and completion behavior.

[INSERT WAVEFORM: Task2_Generic_Timer_Waveform.png]

8.2 Junction Controller Waveform:

Moore FSM sequence and pedestrian handling.

[INSERT WAVEFORM: Task2_Junction_Controller_Waveform.png]

8.3 Two-Junction Waveform:

Junction A and B using the same RTL with different parameters.

[INSERT WAVEFORM: Task2_Two_Junctions_Waveform.png]

9. Coding Practices Followed:

Timing values use parameters rather than magic numbers.

Sequential logic uses non-blocking assignments.

Combinational logic uses blocking assignments.

Case statements contain default branches.

Registers are reset to known safe values.

The traffic controller uses a Moore FSM.

The timer is an independent reusable module.

A non-zero all-red interval is maintained for safety.

10. Corner-Case Verification:

count_target = 0 was tested.

Reset during timer operation was tested.

Different target values were tested without changing timer RTL.

Different timing parameters were used for Junction A and B.

Pedestrian requests were included in simulation.

Safe all-red phases are present between traffic directions.

11. Results and Discussion:

The simulation results demonstrate correct operation of the parameterized RTL architecture. The generic timer can be reused with different targets, while the junction_controller supports different traffic timing requirements. The two-junction integration confirms that the same RTL module can be instantiated multiple times without duplicating the design.

12. Conclusion:

Task 2 successfully implemented a reusable parameterized RTL architecture for an adaptive traffic controller. A generic timer and Moore FSM based junction controller were designed and verified in Vivado. The same controller was reused for two junctions with different parameters, demonstrating modularity, configurability and RTL reusability.

13. Files Used:

generic_timer.v

junction_controller.v

traffic_two_junction.v

generic_timer_tb.v

junction_controller_tb.v

traffic_two_junction_tb.v

14. Tool and Environment:

| Item | Details |
| --- | --- |
| HDL | Verilog HDL |
| Tool | Xilinx Vivado 2022.2 |
| Design | Parameterized RTL / Moore FSM |
| Target | xc7k70tfbv676-1 |